JCAS 5G VEHICULAR NETWORK

VR Laboratory Experiment Guide

Joint Communication and Sensing

Resource Allocation for Autonomous Mobility in 5G Networks

Course:  Wireless / 5G Communications

Lab Type:  Interactive VR Simulation + Optimization Experiment

Duration:  2-3 Hours

# 1. Lab Overview

This VR laboratory experiment allows you to observe and interact with a 5G Joint Communication and Sensing (JCAS) system serving 3 autonomous vehicles in real time. You will use the VR headset to enter a 3D highway environment where you can see radio beams, radar pulses, and data packets — and observe how changing resource allocation affects system performance.

## 1.1 Learning Objectives

By the end of this lab you will be able to:

- Explain what JCAS is and why it matters for autonomous vehicles

- Describe the effect of bandwidth allocation on communication latency

- Describe the effect of beamforming on sensing accuracy

- Read and interpret 5G metrics: SINR, sensing SNR, CRLB error, and TX latency

- Compare optimized vs. equal resource allocation using real simulation data

- Explain the safety constraint and how sensing error affects vehicle safety distance

## 1.2 Equipment Required

- VR Headset (Meta Quest 2/3, HTC Vive, or any WebXR-compatible device)

- Laptop or PC with Chrome/Firefox/Edge browser (latest version)

- Simulation file: jcas_vr.html (provided with this guide)

- This lab guide document

Note: If no VR headset is available, the simulation works in full 3D on any screen. Drag to orbit the camera, scroll to zoom.

# 2. Background

## 2.1 The Real-World Scenario

A 5G base station (gNB) stands at the side of a highway. It serves 3 autonomous vehicles (AVs) simultaneously. The gNB has 4 directional antennas and 20 MHz total radio bandwidth. Every 1 millisecond (TTI), the gNB decides: (1) How much bandwidth to give each car. (2) Where to steer each antenna beam. (3) How much packet error to allow. The same transmitted signal serves both communication (control commands) and sensing (radar obstacle detection). This is JCAS.

## 2.2 Key Terms Reference Card

| Symbol | Name | Plain English Meaning | Value in Lab |
| --- | --- | --- | --- |
| B | Total Bandwidth | Total radio spectrum available, shared among all cars | 20 MHz |
| Ts | Time Slot (TTI) | How often system makes new allocation decisions | 1 ms |
| bu | Bandwidth Fraction | Fraction of 20 MHz given to car u. All must sum to 1. | 0.30-0.38 |
| wu | Beamforming Vector | Controls antenna direction and power toward car u | 4x1 complex |
| gamma_u | SINR | Signal quality. Higher = clearer signal = faster data | 6.7-8.6 dB |
| gamma_s_u | Sensing SNR | Radar quality. Higher = more accurate obstacle sensing | 12.9-15.6 dB |
| epsilon_u | Sensing Error | Best possible range accuracy (CRLB). Lower = more precise | ~20-27 cm |
| tau_u | TX Latency | Time to deliver one control packet. Must be under 1 ms. | <0.04 ms |
| d_safe | Safety Margin | Min distance from obstacle = base + kappa*sensing_error | 5.0-5.2 m |

## 2.3 The Optimization Problem

The gNB solves this every millisecond:

Minimize:    SUM_u [ alpha*latency_u  +  beta*sensing_error_u ]  +  eta*power

Subject to:  total_bandwidth <= 1,   total_power <= Ptotal

each car gets all its bits per slot  (URLLC constraint)

reliability >= 99.999%  (epsilon_u <= 1e-5)

cars maintain safe distance given sensing uncertainty

Solved using SLSQP (Sequential Least Squares Programming) in Python scipy.

# 3. VR Simulation Guide

## 3.1 Loading the Simulation

- Connect your VR headset to the computer

- Open jcas_vr.html in Chrome browser

- You will see a 3D highway with a tower, 3 cars, and animated signals

- Click ENTER VR button (bottom-right) to enter the headset

- Or use mouse drag to orbit the camera on screen

## 3.2 What You See in the VR Scene

- Blue Glowing Tower (left): The gNB base station with 4 antenna elements

- 3 Cars on road: AV1 (cyan), AV2 (orange), AV3 (green) at different speeds

- Colored beam cones: Radio beams from tower to cars. Wider = more bandwidth

- Expanding ring pulses: Radar sensing pulses. Faster/brighter = better sensing SNR

- Flying boxes: Data packets from tower to car. Larger box = more bandwidth

- HUD display (top): Real-time metrics for each allocation mode

## 3.3 The 4 Allocation Modes

| Mode Button | What it Does | Objective | What to Observe |
| --- | --- | --- | --- |
| Equal Allocation | Gives each car exactly 1/3 of resources | 1.067 (poor) | All beams same size |
| Optimized JCAS | Solver finds best split based on each car needs | 0.537 (best safe) | AV2 beam wider |
| High Power | Increased transmit power budget | 0.31 | All beams brighter |
| Resource Starved | AV2 takes 80%: AV1 & AV3 neglected | 2.84 (dangerous) | AV1 & AV3 flash red |

# 4. Experiment Procedure

## Experiment A: Equal vs Optimized Allocation (25 min)

- Load simulation. Start in Equal Allocation mode. Observe for 2 minutes.

- Note beam sizes: are all 3 beams the same size?

- Record HUD values: objective score, average latency, average sensing error.

- Switch to Optimized JCAS mode. Wait 30 seconds.

- Which car has the widest beam? Why? (AV2 sends 600 bits vs 450-500 for others)

- Record the new objective score and calculate the improvement percentage.

- Answer: Is latency better or worse in optimized mode?

Expected finding: Optimized mode reduces objective by ~49.6%. AV2 gets 38.3% bandwidth vs 33.3% because it has more data to deliver (600 bits per slot).

## Experiment B: Safety Constraint Violation (20 min)

- Switch to Resource Starved mode.

- Observe AV1 and AV3 flashing red: this shows a safety constraint violation.

- Record the sensing SNR for AV1 in this mode vs optimized mode.

- Calculate required safety distance: d = 5.0 + 0.5 x epsilon_u(in meters)

- Explain in writing why low sensing SNR forces more vehicle separation distance.

Expected finding: AV1 sensing SNR drops from 14.01 dB to ~3.2 dB in starved mode. This increases sensing error dramatically, forcing a much larger safety buffer distance.

## Experiment C: Power Effect Study (20 min)

- Switch between Equal Allocation and High Power mode repeatedly.

- Observe how the radar pulse rings change: speed, brightness, size.

- Record sensing error in both modes.

- Calculate: what sensing SNR achieves exactly 20 cm accuracy?

- Explain: why does more power lead to better sensing?

# 5. Worksheet

## 5.1 Experiment A Results

| Metric | Equal Mode | Optimized Mode | Change |
| --- | --- | --- | --- |
| Objective Score |  |  |  |
| AV1 Bandwidth % | 33.3% |  |  |
| AV2 Bandwidth % | 33.3% |  |  |
| AV3 Bandwidth % | 33.3% |  |  |
| Avg Latency (ms) |  |  |  |
| Avg Sensing Error |  |  |  |

Q1. Which car has the widest beam in Optimized mode, and why?

Answer: _____________________________________________________________

_____________________________________________________________________

## 5.2 Experiment B Results

| Metric | Optimized Mode | Starved Mode | Notes |
| --- | --- | --- | --- |
| AV1 Sensing SNR | 14.01 dB |  |  |
| AV1 Sensing Error | 23.8 cm |  |  |
| Safety Distance AV1 |  |  | = 5.0 + 0.5*e(m) |
| Objective Score | 0.537 |  |  |

Q2. Explain in your own words why safety constraint links sensing error to vehicle distance:

Answer: _____________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

## 5.3 Experiment C Results

| Metric | Equal Mode | High Power | Your Calculation |
| --- | --- | --- | --- |
| Avg Sensing Error |  |  |  |
| Radar Pulse Speed | Observe & note |  |  |
| Objective Score |  |  |  |

Q3. What sensing SNR (dB) gives exactly 20 cm sensing accuracy?

Formula: epsilon = c/(4*pi*B*sqrt(gamma_s))  =>  gamma_s = [c/(4*pi*B*epsilon)]^2

Show working: _______________________________________________________

Answer:  gamma_s = ________ linear  =  ________ dB

# 6. Key Formulas Summary

## Communication SINR

gamma_u = |h_u^H * w_u|^2  /  ( SUM_{j!=u} |h_u^H * w_j|^2  +  sigma^2 )

## Finite-Blocklength Rate (Polyanskyi 2010)

r_u = log2(1+gamma_u) - sqrt(V_u / m_u) * Q^-1(epsilon_u) / ln(2)

V_u = 1 - (1+gamma_u)^-2      m_u = Ts * B * b_u

## CRLB Sensing Error (Range)

epsilon_u = c / (4 * pi * B * sqrt(gamma_s_u))

## Safety Constraint (Causal, One-Step Ahead)

d(t+1) >= d_safe(v_u) + kappa * epsilon_u(t)

## Optimization Objective

minimize: SUM_u [ alpha_u * tau_u + beta_u * epsilon_u ] + eta * SUM_u ||w_u||^2

# 7. Python Simulation Results

| Metric | AV 1 | AV 2 | AV 3 | Mean |
| --- | --- | --- | --- | --- |
| Bandwidth bu | 0.316 | 0.383 | 0.301 | 0.333 |
| SINR (dB) | 8.60 | 6.71 | 8.55 | 7.95 |
| Sensing SNR (dB) | 14.01 | 15.64 | 12.87 | 14.17 |
| Sensing Error (cm) | 23.78 | 19.71 | 27.13 | 23.54 |
| TX Latency (ms) | 0.027 | 0.032 | 0.025 | 0.028 |
| Error Probability | 1e-5 | 1e-5 | 1e-5 | 1e-5 |

Objective (equal allocation): 1.067

Objective (optimized): 0.537

Improvement: 49.6% -- All URLLC and safety constraints satisfied

# 8. Post-Lab Report Requirements

Submit a written report (1500-2000 words) covering:

- Introduction to JCAS and its role in 5G autonomous vehicle networks

- Explanation of the resource allocation problem: objective and all constraints

- Description of each VR experiment and what you observed

- Analysis of worksheet results with comparison tables

- Discussion of the trade-off between communication quality and sensing accuracy

- Conclusion: What does the 49.6% objective improvement tell us?

Include your completed worksheet tables and at least 3 screenshots from the VR simulation (Equal, Optimized, and Starved modes).